Wire the Boe-Bot as follows: a 220 Ohm resistor from P9 to prong 3 of the IR; prong two (center prong) of the IR to ground (Vss) using a jumper cable wire, and prong 1 of the IR to source (Vdd), using a jumper cable wire.

PBASIC Code:

' IR Using Sony TV Remote Control - SonyRemoteBoeBotControl.bs2

' Use a Sony TV remote to operate the Boe-Bot, using keys 1-8, 4 bit.

'{$STAMP BS2}

'{$PBASIC 2.5}

time VAR Word(4) ' SONY TV remote variables.

FREQOUT 4, 2000, 3000 ' Start/reset indicator tone.

DEBUG "Press and hold remote control key (1-8) to operate Boe Bot..."

DO ' Beginning of main loop.

DO ' Wait for rest between messages.

RCTIME 9, 1, time(0)

LOOP UNTIL time(0) > 1000

PULSIN 9, 0, time(0) ' Measure/store data pulses.

PULSIN 9, 0, time(1)

PULSIN 9, 0, time(2)

PULSIN 9, 0, time(3)

IF (time(3) > 500) THEN

PULSOUT 13, 650

PULSOUT 12, 750

ELSEIF (time(2) < 500) AND (time(1) < 500) AND (time(0) < 500) THEN

PULSOUT 13, 750

PULSOUT 12, 650

ELSEIF (time(2) < 500) AND (time(1) < 500) AND (time(0) > 500) THEN

PULSOUT 13, 850

PULSOUT 12, 650

ELSEIF (time(2) < 500) AND (time(1) > 500) AND (time(0) < 500) THEN

PULSOUT 13, 850

PULSOUT 12, 750

ELSEIF (time(2) < 500) AND (time(1) > 500) AND (time(0) > 500) THEN

PULSOUT 13, 650

PULSOUT 12, 650

ELSEIF (time(2) > 500) AND (time(1) < 500) AND (time(0) < 500) THEN

PULSOUT 13, 750

PULSOUT 12, 750

ELSEIF (time(2) > 500) AND (time(1) < 500) AND (time(0) > 500) THEN

PULSOUT 13, 850

PULSOUT 12, 850

ELSEIF (time(2) > 500) AND (time(1) > 500) AND (time(0) < 500) THEN

PULSOUT 13, 750

PULSOUT 12, 850

ELSEIF (time(2) > 500) AND (time(1) > 500) AND (time(0) > 500) THEN

PULSOUT 13, 650

PULSOUT 12, 850

ENDIF

LOOP ' Repeat main loop.